++ ++9 AUGUST 4, 2026 EV CHARGING INFRASTRUCTURE RIP AND REPLACE PROPOSAL PREPARED FOR Jerome Jenkins Food4Less 3434 Manthey Rd, Stockton, CA 95206; PREPARED BY Alexander Luckett Zero Impact Energy EV Charging Infrastructure Team.

Table of contents

| Section | Title | Page |
| --- | --- | --- |
| 1 | Executive Summary | 3 |
| 2 | Proposal Overview and Project Information | 4 |
| 3 | Historical Usage and Existing Conditions | 5 |
| 3A | Full-History Actuals | 6 |
| 3B | Historical Operating Baseline | 7 |
| 4 | Revenue Projection Methodology | 8 |
| 4A | Key Modeling Assumptions | 9 |
| 5 | 10-Year Revenue and Economic Benefit Projection | 10 |
| 6 | Proposed Charging Configuration | 11 |
| 6A | Charging Equipment Details | 12 |
| 7 | Return on Investment | 13 |
| 7A | Consolidated 10-Year Cashflow | 14 |
| 7B | Charger Revenue Cashflow | 15 |
| 8 | Rip and Replace Scope | 16 |
| 9 | Electrical and Civil Infrastructure Scope | 17 |
| 10 | EVOLV Software and Payment Operations | 18 |
| 11 | Operations and Maintenance | 19 |
| 12 | Warranty and Service Coverage | 20 |
| 13 | Carbon Credits and Environmental Value | 21 |
| 14 | Incentives, Tax Benefits and Financing | 22 |
| 15 | Project Delivery Plan and Schedule | 23 |
| 16 | Detailed Scope of Work | 24 |
| 17 | Financing Options | 25 |
| 17A | Modeled Monthly Cashflow | 26 |
| 17B | 60-Month Financing Cashflow — Months 1 through 25 | 27 |
| 17C | 60-Month Financing Cashflow — Months 26 through 50 | 28 |
| 17D | 60-Month Financing Cashflow — Months 51 through 60 | 29 |
| 18 | Terms, Proposal Validity and Acceptance | 30 |

| 1 | EXECUTIVE SUMMARY |
| --- | --- |

Project recommendation

The Food4Less site has been operating five Level 2 and five Level 3 EV charging ports since August 2023. Over that 35-month period, the station generated $31,220 in annual gross revenue and $13,118 in annual net profit after electricity costs. The constraint is not demand. It is equipment availability. On an average day, only 3.43 of five fast-charging ports and 1.91 of five Level 2 ports were operational, leaving an estimated 28% and 58% of station time out of service, respectively. A further 25.5% of fast-charging sessions and 50.3% of Level 2 sessions delivered less than 1 kWh, indicating drivers arriving, attempting to charge, and leaving without a completed transaction. Among ports that were in service, occupancy reached 57.7% on the fast chargers, which indicates that customer demand at this location already exceeds what the current hardware can reliably serve.

This proposal replaces the existing installation with two Generation 3 APEX SXE DC fast chargers rated at 120 kW, one Generation 2 DC fast charger rated at 60 kW, and ten Flux AC Longhorn Level 2 ports. The system is supported by the EVOLV monitoring and payment platform, U.S.-based service coverage, and industrial-grade hardware built to NEMA 3 and IK10 standards. The modeled DC fast-charging fleet rating is 98 kW per unit. The design raises total port count from ten to thirteen and targets modeled out-of-service time of approximately 3%. On that basis, charging operations alone are projected to produce $53,332 in annual gross revenue and $22,592 in annual net profit, a 72% improvement over current performance, achieved primarily by capturing demand the site is already losing to downtime and failed sessions.

The proposed DC fast chargers add North American Charging Standard, also known as NACS or Tesla, connector capability in place of legacy CHAdeMO cables. NACS began broad industry adoption in 2024 and is increasingly standard on new North American EVs. Adding NACS alongside CCS1 improves compatibility with the expanding vehicle mix and positions the site to serve more drivers over the equipment lifecycle.

Total project cost is $272,023.03 after discount, reduced from a grand total of $315,180.43. Charging revenue is one of three modeled return streams. Over ten years, the project is modeled to generate $406,173 in EVSE revenues, $218,460 in California LCFS carbon credits, and $81,607 in federal investment tax credits, for modeled net returns of $434,217 against the initial outlay. On the consolidated cashflow basis shown in this proposal, the project reaches cumulative break-even during year four and remains profitable for the balance of the ten-year horizon.

Financing is available through De Lage Landen, a Rabobank Group subsidiary. A five-year facility of $272,023.03 at 8.39% produces a fixed monthly payment of $5,566.56 and $61,970.48 in total interest. Under the modeled assumptions, the combination of carbon-credit revenue and charging profit covers the obligation from the first month, producing positive monthly cashflow of approximately $314 at the outset and approximately $1,661 per month by year five.

Proposed solution at a glance

| Component | Configuration | Customer value |
| --- | --- | --- |
| Level 2 charging | 10 single-port 32A units | More usable ports for longer-dwell customers |
| DC fast charging | 2 x 120 kW Gen3 + 1 x 60 kW Gen2 | Faster turnover and higher revenue potential |
| Connectors | CCS1 and NACS capability | Broader compatibility with current and future EVs |
| Software | EVOLV network, payments and reporting | Operational visibility and revenue control |
| Service | Five-year service and warranty allowance in model | Defined support path and lifecycle coverage |
| Delivery | Rip and replace | Reduced disruption and controlled installation scope |

| 2 | PROPOSAL OVERVIEW AND PROJECT INFORMATION |
| --- | --- |

Proposal purpose

This proposal presents a customer-specific modernization plan for the Food4Less EV charging site at 3434 Manthey Rd, in Stockton, California. It combines historical operating performance, recommended equipment, project pricing, revenue projections, service coverage, carbon-credit assumptions, financing, delivery scope and commercial terms in one coordinated offer.

Project information

| Field | Project information |
| --- | --- |
| Client contact | Jerome Jenkins |
| Site | Food4Less |
| Address | 3434 Manthey Rd, Stockton, CA 95206 |
| Property type | Retail |
| Primary users | Customers and public use |
| Proposal date | August 3, 2026 |
| Proposal version | V1.0 |
| Validity | 30 days from proposal date unless extended in writing |

Customer objectives

| Commercial priorities • Attract and retain EV drivers • Generate charging and ancillary site revenue • Replace unreliable or obsolete equipment | Infrastructure priorities • Improve the customer charging experience • Create a scalable platform for future expansion • Support sustainability and operational goals |
| --- | --- |

Proposal contents

| Section | Topic | Section | Topic |
| --- | --- | --- | --- |
| 1 | Executive Summary | 10 | EVOLV Software and Payment Operations |
| 2 | Proposal Overview and Project Information | 11 | Operations and Maintenance |
| 3 | Historical Usage and Existing Conditions | 12 | Warranty and Service Coverage |
| 4 | Revenue Projection Methodology | 13 | Carbon Credits and Environmental Value |
| 5 | 10-Year Revenue and Economic Benefit Projection | 14 | Incentives, Tax Benefits and Financing |
| 6 | Proposed Charging Configuration | 15 | Project Delivery Plan and Schedule |
| 7 | Return on Investment | 16 | Detailed Scope of Work |
| 8 | Rip and Replace Scope | 17 | Financing Options |
| 9 | Electrical and Civil Infrastructure Scope | 18 | Terms, Proposal Validity and Acceptance |

| 3 | HISTORICAL USAGE AND EXISTING CONDITIONS |
| --- | --- |

Site history

The Level 3 chargers bring in most of the revenue on site. At $0.70/kWh retail against $0.40/kWh delivered cost, every kWh carries $0.30 of gross margin — a 42.8% margin on revenue, which is strong for a retail energy business and far better than most public charging operators achieve. Over the full 35-month window that produced roughly $38,000 in net profit from a five-port installation, with no evident price resistance: customers are paying a 35% premium over the Level 2 rate and still choosing the fast chargers overwhelmingly, which suggests speed rather than price drives the decision and that there may be headroom on rate. The hardware also performs close to spec when it's working — 34.42 kW average delivered against a 49 kW derated rating is 70% of nameplate, which is about as good as real-world DC fast charging gets once vehicle-side tapering and battery acceptance limits are accounted for.

Level 3 sees 5.43 sessions per day and 2.15 of five stalls in use daily — 43% raw occupancy, or 57.7% counting only stalls that were actually alive. More telling is the shape of the distribution: the median Level 3 session (18.88 kWh) sits close to the mean (21.99 kWh), meaning revenue comes from a broad base of substantial, similar-sized charges rather than a handful of outliers propping up the total. That is the profile of predictable, forecastable income. Short dwell times help too — 38 minutes per session means high stall turnover, so each functioning port serves many customers per day rather than being tied up by one.

| 35 months HISTORY REVIEWED Aug 2023 to Jun 2026 | $92,358 HISTORICAL REVENUE Level 2 + Level 3 actual | 96% DC FAST REVENUE SHARE $88,883 of total | $6,953 ESTIMATED OUTAGE LOSS Jan and Feb 2026 |
| --- | --- | --- | --- |

Historical monthly charging revenue, August 2023 through June 2026

| 3A | FULL-HISTORY ACTUALS Continuation of Section 3 |
| --- | --- |

| FULL-HISTORY ACTUALS (Aug 2023 – Jun 2026) | Level 2 | Level 3 |
| --- | --- | --- |
| Months in window | 35 |  |
| Days in window | 1065 |  |
| Total revenue | $3,474.82 | $88,883.41 |
| Total kWh | 6,693.9 | 127,182.3 |
| Avg revenue per month | $99.28 | $2,539.53 |
| Avg kWh per month | 191.3 | 3,633.8 |
| Avg kWh per day | 6.29 | 119.42 |
| Effective retail rate ($/kWh) | $0.519 | $0.699 |
| Avg stalls used per day | 0.766 | 2.152 |
| Avg charging hours per active stall per day | 3.157 | 1.612 |
| Avg delivered power while charging (kW) | 2.60 | 34.42 |
| Total sessions | 1234 | 5782 |
| Sessions per day | 1.16 | 5.43 |
| Avg kWh per session | 5.42 | 21.99 |
| Median kWh per session | 0.97 | 18.88 |
| Sessions delivering <1 kWh (failed/aborted) | 50.3% | 25.5% |
| Avg charging time per session (min) | 125.22 | 38.34 |
| Avg stalls in service per day (hardware alive) | 1.91 | 3.43 |
| Stall occupancy among IN-SERVICE stalls | 24.4% | 57.7% |

| 3B | HISTORICAL OPERATING BASELINE Continuation of Section 3 |
| --- | --- |

| Metric | Level 2 | Level 3 (Fast) |
| --- | --- | --- |
| Chargers | 5 | 5 |
| EV Stall Quantity (Based on # of ports) | 5 | 5 |
| Max hours of operation of parking | 12 | 12 |
| Stall Occupancy % (stalls seeing use each day) | 15.3% | 43.0% |
| Number of stalls used per day | 0.77 | 2.15 |
| Charging Hourly % per stall per day | 26.3% | 13.4% |
| Number of hours usage per stall per day | 3.16 | 1.61 |
| Charger Rating (kW, incl. 0.98 de-rate) | 7.06 | 49.00 |
| Avg Delivered Power (% of rating) | 36.8% | 70.3% |
| Retail Revenue per kWhr | $0.519 | $0.699 |
| Total Revenue per day | $3.26 | $83.46 |
| Per 30 day Cycle | $97.88 | $2,503.76 |
| Total kWhr dispensed per day | 6.3 | 119.4 |
| EV Utility Rate average per kWhr | $0.40 | $0.40 |
| Net Profit per Month | $22.46 | $1,070.72 |
| Total Profit per Year | $269.50 | $12,848.62 |
| Estimated share of station time out of service | 58% | 28% |
| TOTAL NET PROFIT (revenue − electricity @ $0.40/kWh) | Overall Net Profit | Overall Net Profit |
| Yearly | $13,118 | $13,118 |
| 3 Years | $39,354 | $39,354 |
| 5 Years | $65,591 | $65,591 |
| Gross Revenue (Yearly, before electricity) | $31,220 | $31,220 |
| DATA BASIS: Historical performance is based on recorded site activity from August 2023 through June 2026. Actual future performance will depend on equipment availability, utility rates, pricing, vehicle mix, driver demand and operating policy. |  |  |

| 4 | REVENUE PROJECTION METHODOLOGY |
| --- | --- |

Forecast structure

| Layer | Calculation logic | Client-facing output |
| --- | --- | --- |
| 1. Historical actual | Observed sessions, kWh, revenue and uptime | Actual baseline with source period |
| 2. Operational normalization | Adjust for dead equipment, outages, failed starts and partial periods | Normalized demand and recoverable usage |
| 3. Proposed configuration | Ports x occupancy x hours x delivered kW x retail price | Projected charging revenue and kWh |
| 4. Operating cost | Projected kWh x utility energy rate plus network and O&M costs | Net operating profit |
| 5. Idle policy | Sessions x overstay share x billable minutes x fee | Separate idle-fee revenue |
| 6. Growth scenarios | Conservative, base and upside assumptions by year | Range, not a single guaranteed forecast |
| 7. Non-operating benefits | Tax incentives, grants and environmental credits | Separate, eligibility-qualified benefit lines |

Modeled annual operating profit plus idle fees, Years 1 through 10

| 4A | KEY MODELING ASSUMPTIONS Continuation of Section 4 |
| --- | --- |

| Assumption | Workbook value | Customer-facing treatment |  |
| --- | --- | --- | --- |
| Site operating window | 12 hours per day | Subject to confirmation of actual site access hours |  |
| Utility energy cost | $0.40 per kWh | Subject to current tariff and demand-charge review |  |
| Level 2 retail price | $0.65 per kWh projected | Subject to final pricing policy and applicable taxes or fees |  |
| Level 3 retail price | $0.70 per kWh projected | Subject to final pricing policy and market position |  |
| New-equipment downtime | 3% model assumption | Planning assumption, not a contractual service-level guarantee |  |
| Annual growth | 12.5% in Years 2 through 10 | Base-case growth assumption used in the model |  |
| Idle policy | 60-minute Level 2 / 30-minute Level 3 grace | Subject to final customer-experience and operating policy |  |
| MODEL SCOPE: The 10-year total in this proposal includes operating profit, modeled idle fees and the Year 1 federal investment tax credit assumption. It excludes environmental credits and does not subtract project cost. |  |  |

| 5 | 10-YEAR REVENUE AND ECONOMIC BENEFIT PROJECTION |
| --- | --- |

| $26,870 YEAR 1 OPERATING PROFIT Charging profit + idle fees | $108,477 YEAR 1 TOTAL INCL. ITC Includes modeled federal ITC | $406,173 10-YEAR OPERATING PROFIT Before idle fees | $564,696 10-YEAR TOTAL BENEFIT Operating profit + idle fees + ITC |
| --- | --- | --- | --- |

Projected new-charger operating profile

| Metric | Level 2 | Level 3 (Fast) |
| --- | --- | --- |
| Chargers | 10 | 3 |
| EV Stall Quantity (Based on # of ports) | 10 | 3 |
| Max hours of operation of parking | 12 | 12 |
| Stall Occupancy % (stalls seeing use each day) | 24.4% | 57.7% |
| Number of stalls used per day | 2.44 | 1.73 |
| Charging Hourly % per stall per day | 26.3% | 13.4% |
| Number of hours usage per stall per day | 3.16 | 1.61 |
| Charger Rating (kW, incl. 0.98 de-rate) | 7.53 | 98.00 |
| Avg Delivered Power (% of rating) | 36.8% | 70.3% |
| Retail Revenue per kWHr | $0.650 | $0.699 |
| Total Revenue per day | $13.88 | $134.26 |
| Per 30 day Cycle | $416.44 | $4,027.89 |
| Total kWhr dispensed per day | 21.4 | 192.1 |
| EV Utility Rate average per kWHr | $0.40 | $0.40 |
| Net Profit per Month | $160.17 | $1,722.50 |
| Total Profit per Year | $1,922.03 | $20,670.04 |
| Estimated share of station time out of service | 3% | 3% |
| TOTAL NET PROFIT (revenue − electricity @ $0.40/kWh) | Projected (New Chargers) | Projected (New Chargers) |
| Yearly | $22,592 |  |
| 3 Years | $67,776 |  |
| 5 Years | $112,960 |  |
| Gross Revenue (Yearly, before electricity) | $53,332 |  |

Comparison to historical baseline

| Metric | Historical baseline | Projected result | Difference |
| --- | --- | --- | --- |
| Annual operating profit | $13,118 | $26,870 | $13,752 |
| 10-year baseline / benefit | $131,181 | $564,696 | $433,515 |
| Operating uplift | — | 104.8% | Includes modeled idle fees |

| 6 | PROPOSED CHARGING CONFIGURATION |
| --- | --- |

Recommended equipment schedule

| Equipment | Qty | Nameplate rating |  | Modeled rating | Primary use |  |
| --- | --- | --- | --- | --- | --- | --- |
| 32A single commercial Level 2 charging unit | 10 | 7.68 kW each |  | 7.53 kW after 0.98 factor | Longer-dwell customer charging |  |
| Gen3 dual-connector DC fast charging unit | 2 | 120 kW each |  | Included in 98 kW fleet average | High-turnover fast charging |  |
| Gen2 dual-connector DC fast charging unit | 1 | 60 kW |  | Included in 98 kW fleet average | Fast charging and load balancing |  |
| Pedestals, brackets and cable management | As scheduled | N/A |  | N/A | Safe and organized installation |  |

Configuration narrative

The proposed configuration modernizes a proven retail charging location with higher-power DC fast charging, expanded Level 2 capacity, improved connector compatibility and a unified operating platform. The site will retain the customer convenience of both fast-turnover and longer-dwell charging while addressing the downtime and failed-session patterns visible in the historical data.

Being located along the I-5 Fwy, and only a 10-minute drive from the Stockton Airport, your site is one of the largest Level 3 charging sites in the Greater Stockton area and brings in thousands each month through your current older charger models.

Modernizing your chargers will further establish your site as a reliable charging destination as EV charging technology evolves, increasing traffic and customer satisfaction.

As the I-5 Fwy is one of the most driven roads along the West Coast, tourists and city folks combined will see your DC fast chargers as the most efficient place to stop and charge their vehicle while they grab a bite to eat and continue on with their travels.

| FINAL SUBMITTAL: Final connector type, cable length, payment hardware, certifications, enclosure rating, communications, accessibility requirements and product warranty will be confirmed in the approved equipment submittal. |  |
| --- | --- |
| 6A |  | CHARGING EQUIPMENT DETAILS Continuation of Section 6 |  |

Level 2 charging

| Specification | Proposal field |
| --- | --- |
| Model / SKU | CTX-C32-240 |
| Quantity / ports | 10 single-port units |
| Electrical rating | 32A |
| Power | 7.53 kW modeled |
| Mounting / cable management | Pedestal and cable-management configuration as scheduled |
| Payment and access | EVOLV app and guest payment |

DC fast charging

| Specification | Proposal field |
| --- | --- |
| Models / quantities | 2 x Generation 3 120 kW and 1 x Generation 2 60 kW |
| Modeled fleet rating | 98 kW average |
| Connector configuration | CCS1 and NACS / dual connector as selected |
| Payment | Integrated payment method and EVOLV configuration |
| Cable management | Cable retractors on each Level 3 cord |

NACS connector addition

The modernization replaces legacy CHAdeMO capability with NACS / Tesla connector capability on the Level 3 chargers while retaining CCS1 as selected. This expands access for a growing share of North American EVs and reduces reliance on adapters for compatible vehicles.

| 7 | RETURN ON INVESTMENT |
| --- | --- |

Initial investment

With an initial investment of $272,023.03, we project a break-even point in year 4, followed by full profits for the next 6 years. By installing the new Generation 3 APEX SXE DC Fast Chargers and Flux AC Longhorn chargers, your site will earn more than just charger revenue. In addition to the increased charger revenue, you will also be eligible to receive Carbon Credits, and Incentive Tax Credits back as well. Below is the breakdown of how the return on investment would be over the next 10 years.

Initial investment costs

| Cost category | Customer price |
| --- | --- |
| Equipment Purchase Invoice (Finance) | $225,731.62 |
| Charger Hardware | $144,971.75 |
| 5 Year Service Agreement & Warranty | $46,560.09 |
| EVOLV & Commissioning Subtotal | $21,834.54 |
| Sales Tax on Chargers | $12,365.24 |
| Electrical Supply & Construction Management Costs | $21,541.41 |
| Wires, Conduits and Peripherals | $3,575.67 |
| Main Distribution Switchgear | $0.00 |
| Electrical Sub-Panels, Transformers, Breakers | $2,178.00 |
| Striping, Bollards, Signage | $2,895.17 |
| Concrete Improvements | $2,841.10 |
| Dump/ Waste | $2,420.00 |
| Construction Equipment | $7,631.47 |
| Labor | $24,750.00 |
| Grand Total | $315,180.43 |
| Total After Discount | $272,023.03 |

Projected ROI after 10 years

| Projected ROI over 10 years | Value |  |
| --- | --- | --- |
| Total Costs Upfront | -$272,023.03 |  |
| Carbon Credits | $218,459.66 |  |
| Federal ITC (30%) | $81,606.91 |  |
| EVSE Revenues | $406,173.10 |  |
| Net Revenues | $434,216.64 |  |
| QUALIFICATION: Modeled tax incentives and environmental credits are subject to eligibility, documentation, program rules, credit ownership, market pricing and applicable tax advice. |  |

| 7A | CONSOLIDATED 10-YEAR CASHFLOW Continuation of Section 7 |
| --- | --- |

How the model calculates break-even

With the initial investment of $272,023.03, we project a steady increase in annual revenues. Using the following consolidated cashflow, we expect a break-even point during year 4, with only profits following for the next 6 years.

| Year | Annual cashflow | Cumulative |
| --- | --- | --- |
| 0 | -$272,023 | -$272,023 |
| 1 | $126,045 | -$145,978 |
| 2 | $47,262 | -$98,716 |
| 3 | $50,439 | -$48,277 |
| 4 | $54,013 | $5,736 |
| 5 | $79,880 | $85,616 |
| 6 | $62,558 | $148,174 |
| 7 | $67,647 | $215,820 |
| 8 | $73,372 | $289,192 |
| 9 | $79,812 | $369,004 |
| 10 | $87,058 | $456,063 |

Modeled cumulative project cashflow

| 7B | CHARGER REVENUE CASHFLOW Continuation of Section 7 |
| --- | --- |

| Year | Annual cashflow | Cumulative |  |
| --- | --- | --- | --- |
| 0 | -$272,023 | -$272,023 |  |
| 1 | $22,592 | -$249,431 |  |
| 2 | $25,416 | -$224,015 |  |
| 3 | $28,593 | -$195,422 |  |
| 4 | $32,167 | -$163,255 |  |
| 5 | $36,188 | -$127,066 |  |
| 6 | $40,712 | -$86,355 |  |
| 7 | $45,801 | -$40,554 |  |
| 8 | $51,526 | $10,971 |  |
| 9 | $57,966 | $68,938 |  |
| 10 | $65,212 | $134,150 |  |
| TABLE SCOPE: This table isolates modeled charger operating revenue and does not include carbon credits, federal tax benefits or other project value streams. |  |  |

| 8 | RIP AND REPLACE SCOPE |
| --- | --- |

This scope applies to removal and replacement of the existing charging equipment and integration with reusable infrastructure where field conditions allow. Confirmed reuse will be distinguished from items that remain subject to site verification and final engineering.

Rip-and-replace activities

| Phase | Included activities | Key client decision / dependency |
| --- | --- | --- |
| Site verification | Document existing equipment, conductors, breakers, panels, pads, bollards, network and ADA conditions | Access to site, as-builts and shutdown windows |
| Decommissioning | Lockout/tagout, disconnect, remove and stage existing charging equipment | Ownership, disposal and data-retention instructions |
| Reuse assessment | Test conductors, grounding, breakers, panels, conduit, foundations and communications | Approval of any reuse-related risk or exclusions |
| Retrofit work | Modify pads, conduit, conductors, protection, bollards, striping and signage as required | Final engineering and permitting |
| Installation and cutover | Install charging equipment, configure network and payment systems, test and energize | Utility availability and coordinated outage plan |
| Closeout | Commissioning report, photos, serial numbers, as-builts, training and warranty registration | Client acceptance and punch-list closure |

Removal and disposal

Removed equipment will be handled in accordance with the final contract, including client direction regarding retention, recycling, return to the original equipment manufacturer, storage or disposal. Any data-retention or network decommissioning requirements will be completed before final removal.

Downtime and cutover plan

Zero Impact Energy will coordinate the sequence of decommissioning, installation, networking, testing and commissioning to reduce disruption. The final outage window, work hours, partial-service availability and customer communications will be confirmed after site verification, utility coordination and approved construction scheduling.

| 9 | ELECTRICAL AND CIVIL INFRASTRUCTURE SCOPE |
| --- | --- |

Modeled cost categories

| Category | Modeled amount | Description |  |
| --- | --- | --- | --- |
| Wires, conduits and peripherals | $3,576 | Conductors, conduit and associated electrical materials |  |
| Sub-panels, transformers and breakers | $2,178 | Distribution and protection equipment |  |
| Striping, bollards and signage | $2,895 | Parking and equipment-protection improvements |  |
| Concrete improvements | $2,841 | Pads, repairs or associated concrete work |  |
| Dump / waste | $2,420 | Removal and disposal allowance |  |
| Construction equipment | $7,631 | Equipment and mobilization allowance |  |
| Other / miscellaneous | $0 or project-specific | Permits, utility, paving, ADA and additional materials as applicable |  |
| SCOPE BASIS: Permitting, utility, engineering, paving and ADA work are included only to the extent expressly stated in the final scope and pricing. Additional requirements identified during design or field verification may require a written change order. |  |  |

| 10 | EVOLV SOFTWARE AND PAYMENT OPERATIONS |
| --- | --- |

EVOLV is the charging operations platform used to connect charging assets, manage driver access and payments, monitor performance and provide operating data. The platform configuration distinguishes software functions from field O&M services so the client understands what the platform does and what requires technician support.

| Core platform functions • Real-time charger status and fault visibility • Driver access, payment and session management • Pricing, grace-period and idle-fee configuration • Utilization, energy, revenue and settlement reporting • User, site and portfolio administration • Open-protocol integration subject to equipment compatibility | Commercial configuration • Number of ports and platform term • Per-port monthly or annual fee • Payment-processing and guest-payment fees • Merchant-of-record and settlement timing • Data ownership, retention and reporting access • Support hours and escalation process |
| --- | --- |

Modeled platform economics

| Item | Modeled value | Commercial treatment |
| --- | --- | --- |
| Network cloud service fee | $39.99 per port per month | Confirm current rate and included functions |
| Ports | 13 | Must match final installed and active port count |
| Annual platform cost | $6,238 | 13 x $39.99 x 12 |
| Contract duration | 5 years | Subject to final contract term, renewal and termination rights |
| Modeled customer platform total | $21,835 | Reflected in current project pricing |

Charger support and service

Rapid remote diagnostics and issue resolution

Localized U.S. support and consistent escalation

Real-time system monitoring and performance visibility

Phone and email support channels

Extended service hours from 7 a.m. to 10 p.m.

Tailored operator training sessions of approximately 2 to 3 hours

| 11 | OPERATIONS AND MAINTENANCE |
| --- | --- |

What O&M means

Operations and maintenance is the ongoing process of keeping the charging site available, safe, connected, properly configured and economically productive after commissioning. O&M is broader than a parts warranty and broader than software monitoring. It combines remote operations, issue management and field service under a defined support model.

| O&M function | What it includes | Current treatment |  |
| --- | --- | --- | --- |
| Remote monitoring | Status review, alerts, diagnostics and ticket creation | Included through EVOLV and selected service plan |  |
| Remote resolution | Resets, configuration, firmware and network troubleshooting | Included during support hours |  |
| Preventive maintenance | Inspection, cleaning, torque checks, filters, cables and safety review | Per final service plan and checklist |  |
| Corrective maintenance | Technician dispatch, diagnosis and repair | Per final service plan and response target |  |
| Parts logistics | RMA, spare parts, shipping and replacement coordination | Subject to warranty and parts coverage |  |
| Performance reporting | Uptime, faults, sessions, revenue and recurring-issue analysis | Monthly or quarterly as selected |  |
| Customer support | Driver or site-host support channels and escalation | Phone, email and platform channels |  |
| SERVICE FRAMEWORK: O&M is broader than the equipment warranty. Final responsibilities, response targets, preventive-maintenance frequency, labor coverage and exclusions will be governed by the executed service agreement. |  |  |

| 12 | WARRANTY AND SERVICE COVERAGE |
| --- | --- |

The project separates manufacturer equipment warranty, Zero Impact Energy service obligations, EVOLV platform support and any extended coverage. The final contract will identify parts, labor, shipping, field response, exclusions and the process for requesting service.

| Coverage layer | Typical responsibility |  | Customer-facing treatment |  |
| --- | --- | --- | --- | --- |
| Manufacturer parts warranty | OEM replacement of eligible failed components |  | Term, covered parts, exclusions, RMA and shipping stated in final warranty |  |
| Labor warranty | Installer correction of covered workmanship issues |  | Term, covered work and exclusions stated in contract |  |
| O&M service plan | Monitoring, preventive and corrective service |  | Hours, response targets, labor, travel and parts limits stated in service plan |  |
| Software support | Network and application support |  | Support hours, channels and escalation stated in EVOLV terms |  |
| Extended warranty | Additional equipment coverage after standard term |  | Price, term, deductible and exclusions stated if selected |  |
| 2 years STANDARD PARTS WARRANTY Equipment basis | 5 years MODELED SERVICE TERM Subject to final agreement | $46,560 SERVICE / WARRANTY ALLOWANCE Included in customer price |  | $24,750 LABOR ALLOWANCE Current modeled amount |
| COVERAGE CONTROL: Final coverage is governed by the executed manufacturer warranty, service agreement and project contract. Where descriptions differ, the signed documents control. |  |  |  |  |

| 13 | CARBON CREDITS AND ENVIRONMENTAL VALUE |
| --- | --- |

There are two main types of Carbon Credits. Consumption (usage-based) credits are earned on every kWh your chargers actually dispense—roughly one metric ton of credits for every 3,500–4,500 kWh delivered—so they grow with site usage and apply to both Level 2 and Level 3 chargers. Capacity credits, formally called Fast Charging Infrastructure (FCI) credits, instead reward the charging capacity you install regardless of whether it's used. FCI applies only to public DC fast chargers (Level 3) rated 50 kW or more that were permitted on or after January 1, 2022 and are networked with real-time uptime reporting. FCI is designed to give sites predictable revenue for up to 10 years during their low-traffic early years, though it's first-come, first-served, capped statewide at 2.5% of California's fuel deficits, and a site stops qualifying for capacity credits once cumulative FCI revenue exceeds 1.5x the net cost of its capital expenditures (after grants).

Most site owners don't deal with CARB directly. They work with a registered aggregator (such as FuSE or Xpansiv) that handles charger registration, quarterly data reporting, verification, and credit sales, keeping a percentage of the revenue in exchange. Credit value ultimately depends on charger type, energy dispensed, reliable uptime, and accurate documentation—so the credits can meaningfully improve project economics, operating costs, maintenance, and future expansion, but only when the site is eligible and well-managed.

Credit values are calculated differently depending on charger type. Level 3 (fast) chargers are valued per unit according to their power rating, using a pre-calculated annual credit yield applied across the term of the agreement and breaks down to the number of chargers multiplied by the annual credit value multiplied by the number of years with current rates of $8,600 per year for the 100 kW class. Level 2 chargers, by contrast, are metered on actual energy consumption rather than a fixed per-unit rate, so the total kilowatt-hours delivered must be estimated from occupancy and charging speed: estimated monthly kWh multiplied by the credit value per kWh ($0.0045) multiplied by the number of months, where monthly kWh is derived as ports × occupancy rate × charging hours × 6.9 kW.

Required commercial terms

| Topic |  | Proposal requirement |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Program and eligibility |  | Identify the specific program and confirm the site and charging activity are eligible |  |  |  |
| Credit ownership |  | State whether the client, Zero Impact Energy, utility, financier or aggregator owns the credits |  |  |  |
| Registration and reporting |  | Identify who registers equipment, submits data and maintains records |  |  |  |
| Revenue sharing |  | State gross credit value, aggregator fees and net customer share |  |  |  |
| Price and volume assumptions |  | Show the assumed credit price, energy volume and sensitivity |  |  |  |
| Timing and risk |  | Explain issuance lag, market-price volatility, rule changes and non-guarantee |  |  |  |
| $218,460 MODELED 10-YEAR CREDITS Workbook estimate | $8,600 / $4,300 LEVEL 3 MULTIPLIERS Fixed model inputs |  | $0.0045 LEVEL 2 FACTOR Applied to consumption | Subject to validation ELIGIBILITY STATUS Not guaranteed |  |  |
| PROGRAM QUALIFICATION: The modeled carbon-credit amount is not contracted revenue. Eligibility, methodology, ownership, aggregator fees, pricing and reporting responsibilities must be documented before credits are treated as a committed customer benefit. |  |  |  |  |

| 14 | INCENTIVES, TAX BENEFITS AND FINANCING |
| --- | --- |

Potential capital-stack components

| Component | How to present it | Required qualification |
| --- | --- | --- |
| Utility or public incentive | Estimated award and eligible cost categories | Subject to application, reservation, funding and compliance |
| Federal tax credit | Modeled percentage and eligible basis | Taxpayer eligibility and tax counsel confirmation |
| Environmental credits | Projected operating-period benefit | Program eligibility, ownership and market pricing |
| Equipment financing | Term, rate, monthly payment and total interest | Lender approval, documentation and final pricing |
| Customer capital | Upfront contribution and payment milestones | Contractual payment schedule |

Illustrative financing example

| $272,023 FINANCED AMOUNT Modeled full project | 8.39% INTEREST RATE Model input | 60 months TERM Five years | $5,567 MONTHLY PAYMENT Total interest $61,970 |
| --- | --- | --- | --- |

Illustrative tax-credit history

When Congress passed the OBBBA in July 2025, it stripped the ITC from most technologies but deliberately spared batteries. After the passage of the One Big Beautiful Bill Act, 48E now applies exclusively to standalone energy storage projects — not to solar or wind as it once did. The timeline is generous: battery projects that begin construction before the end of 2033 still qualify for a 30% credit under Section 48E, after which the rate tapers down. So while solar and wind are racing a 2027 cliff, storage has most of a decade of runway.

The mechanics for a business: the base credit is 6 percent of eligible project costs, but it can increase to 30 percent if prevailing wage and apprenticeship requirements are met. On top of that, bonus adders can significantly increase your credit value — a +10% domestic content bonus when U.S. sourcing thresholds are met, a +10% energy community bonus for projects in qualifying areas, and a low-income bonus available through a competitive allocation program for qualifying projects under 5 MW. Critically for a charging site, unlike the prior version of the ITC, this credit does not require the storage system to be directly connected to or charged by a solar system — standalone batteries qualify on their own.

TAX QUALIFICATION: Tax-credit treatment is subject to taxpayer eligibility, placed-in-service timing, prevailing-wage and apprenticeship requirements, domestic-content rules, eligible-basis review and independent tax advice.

| 15 | PROJECT DELIVERY PLAN AND SCHEDULE |
| --- | --- |

| Phase | Typical duration | Key activities | Schedule dependencies |
| --- | --- | --- | --- |
| 1. Validation | 1-3 weeks | Site walk, data review, equipment confirmation and commercial alignment | Site access and complete data |
| 2. Engineering / utility | 4-16+ weeks | Design, utility coordination, permit package and plan check | Utility and AHJ response |
| 3. Procurement | 6-20+ weeks | Equipment release, submittals and material procurement | Final design, deposit and availability |
| 4. Construction | 1-8+ weeks | Civil, electrical, equipment installation and restoration | Permit, utility readiness and site access |
| 5. Commissioning | 2-5 days | Networking, payment setup, testing, training and acceptance | Energization and communications |
| 6. Operations | Ongoing | Monitoring, reporting, maintenance and optimization | Executed service agreement |

Milestone schedule

| Milestone | Target date | Owner | Status / notes |  |
| --- | --- | --- | --- | --- |
| Proposal approval | Upon client authorization | Client / Zero Impact Energy | Pending |  |
| Site verification complete | To be scheduled | Zero Impact Energy | Pending |  |
| Design submitted | After site verification | Engineering | Pending |  |
| Permit / utility approval | Agency dependent | AHJ / utility | Pending |  |
| Equipment released | After approval and deposit | Zero Impact Energy | Pending |  |
| Construction complete | Per final project schedule | Zero Impact Energy / contractor | Pending |  |
| Commissioning and acceptance | Following installation | Joint | Pending |  |
| SCHEDULE BASIS: Schedule durations are planning ranges. The definitive schedule will be issued after utility requirements, permits, equipment availability, site conditions and client access constraints are confirmed. |  |  |  |

Construction timeline

Due to the nature of this project, and the very slight relocation of the chargers, we are expecting this construction to take 3 to 4 weeks. The following Gantt chart shows the estimated timeline for completion.

| Construction Schedule |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Duration | Week 1 |  |  |  |  | S | S | Week 2 |  |  |  |  | S | S | Week 3 |  |  |  |  | S | S |
| Remove Chargers | 1 day |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Demo Concrete Pads | 2 days |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Trenching for Conduit | 2 days |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Rerouting Conduits | 2 days |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Forming Pads | 2 days |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Rebar | 1 day |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Pour Concrete | 1 day |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Install New L3's | 1 day |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Pull Wire | 2 days |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Power up and commission | 1 day |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Estimated construction sequence, shown in working days across four weeks

| 16 | DETAILED SCOPE OF WORK |
| --- | --- |

| Work package | Zero Impact Energy scope | Client / third-party scope |
| --- | --- | --- |
| Project management | Coordinate project team, schedule, submittals, meetings and reporting | Provide timely decisions, access and a designated point of contact |
| Site verification | Field review of visible conditions and available documents | Provide as-builts, utility bills, title or site restrictions and known conditions |
| Engineering | Electrical, civil and structural design as stated in final scope | Review and approve design criteria; provide owner standards |
| Utility / permitting | Applications, coordination and plan check as stated in final scope | Execute owner agreements and pay owner-direct fees as stated |
| Procurement | Supply approved charging equipment and listed accessories | Approve substitutions and release milestones |
| Construction | Perform stated demolition, civil, electrical and restoration work | Provide work area, shutdown approval and site coordination |
| Software / payments | Configure EVOLV, pricing, access and reporting | Provide merchant, banking, branding and policy information |
| Commissioning | Test equipment, network, payment and basic operations | Attend acceptance and identify authorized users |
| Training / closeout | Provide operational training and closeout records | Participate in training and maintain site contacts |
| O&M | Provide the selected support package after acceptance | Provide access, report damage and follow operating procedures |

Charger and battery-storage integration

When connecting chargers into currently existing battery storage systems, there are two options on how to tie the chargers into the system.

Grid-parallel operation

Grid-parallel operation is the configuration that accounts for nearly all real installations, and it's where the economics actually live. The charger draws from the grid as its primary source, and the battery contributes during high-demand windows — shaving the peak rather than carrying the load. This solves two distinct problems. First, it can prevent a utility service upgrade: under NEC 705.13, a listed Power Control System can be used to cap total current on the service so a 200-amp panel can host chargers whose nameplate draw would otherwise exceed it, with the battery and the PCS jointly ensuring the service never gets overloaded. Avoiding a transformer upgrade and the associated utility timeline is frequently worth more than the energy savings. Second, it attacks demand charges, which in California commercial rates can run $20 to $35 per kW and are set by a single 15-minute peak. A 150 kW fast charger firing once during the monthly peak window can add several thousand dollars to a bill; a battery that discharges into that spike flattens it. Worth checking the rate schedule first, though — PG&E's BEV rates and SCE's TOU-EV-8/9 already suppress or eliminate demand charges for qualifying EV loads, which can substantially shrink the arbitrage the battery was going to capture.

Battery-only operation

Battery-only operation — the charger fed exclusively from storage, with no grid contribution during the charging session — is technically achievable but constrained in ways that are easy to underestimate. Two hard requirements: the inverter must be grid-forming, not merely grid-following, and the site needs a microgrid interconnect device or transfer switch to cleanly separate from the utility. Most commercial storage inverters sold as grid-tied are grid-following only; they synchronize to an existing AC waveform and shut down when it disappears. Such a system cannot power a charger during an outage at all, regardless of state of charge. This is the single most common surprise in these projects, and it's worth confirming from the inverter's listing before designing anything around it.

| 17 | FINANCING OPTIONS |
| --- | --- |

De Lage Landen financing

De Lage Landen (DLL), is a wholly owned Rabobank Group subsidiary established in 1969 and based in Eindhoven, the Netherlands. It is an international asset finance organization with an active portfolio exceeding €47 billion across more than 25 countries. The company's model centers on vendor finance, meaning it embeds itself in the supply chain rather than competing for retail borrowers: DLL builds asset-based financing programs with manufacturers, dealers, distributors, and resellers, delivered at their points of sale and structured to follow equipment across its entire technical life. Coverage spans through agriculture, construction, energy transition, food, healthcare, industrial, technology, transportation, and workplace equipment. DLL holds credit institution status under the EU's Capital Requirements Regulation and delivers business through country-level entities while serving several European markets through branches under its passporting rights. Electrification has become a visible strategic priority: DLL works with Zero Impact Energy to provide their clients with the option to finance their clean energy construction process.

DLL schedule

The loan amortization schedule for the project financing. An inputs section defines the terms of the facility: a financed amount of $272,023.03, an annual interest rate of 8.39%, a five-year term, monthly payments (12 per year), and a loan start date of August 1, 2026. From these inputs the sheet calculates a fixed scheduled monthly payment of $5,566.56 across 60 payments, and a total interest cost of $61,970.48 over the life of the loan. The month-by-month table then runs the full 60-payment term — from the first payment in September 2026 through payoff in August 2031 — detailing, for each period, the beginning balance, the split of the payment between principal and interest, the ending balance, and cumulative interest paid to date. Because the payment amount is fixed, the interest portion is largest in the early payments and declines steadily as the principal is retired, with the balance reaching zero at the final payment. This sheet establishes the fixed monthly obligation of $5,566.56 that is carried through the Cashflow analysis, and it confirms the total financing cost — roughly $62,000 in interest on top of the $272,023 principal — that the project's returns must cover.

| Facility input | Value |
| --- | --- |
| Loan amount | $272,023.03 |
| Annual interest rate | 8.39% |
| Loan period | 5 years |
| Payments per year | 12 |
| Modeled start date | August 1, 2026 |
| Optional extra payments | None modeled |
| Loan summary | Value |
| Scheduled payment | $5,566.56 |
| Scheduled number of payments | 60 |
| Actual number of payments | 60 |
| Total early payments | None modeled |
| Total interest | $61,970.48 |

| 17A | MODELED MONTHLY CASHFLOW Continuation of Section 17 |
| --- | --- |

The monthly cashflow view across the full 60-month financing term. Each month sets the fixed $5,566.56 loan payment against the project's two recurring income streams — carbon credit revenue of $3,640.99 per month, held constant throughout the term, and EVSE charging profit — to arrive at a net monthly position. Charging profit is modeled to grow rather than hold flat, stepping up with assumed utilization from roughly $2,239.19 per month in the first year to approximately $2,519.09, $2,833.98, $3,188.22, and $3,586.75 per month in each successive year. With the debt service fixed and charging profit rising, the net position strengthens over the term, moving from about $314 per month at the outset to roughly $1,661 per month by year five. Across all 60 months, the streams total approximately $333,993.51 in loan payments, $218,459.66 in carbon credit revenue, and $172,406.75 in charging profit, leaving cumulative net cashflow of roughly $56,900 over the financing period.

FINANCING QUALIFICATION: Financing is subject to lender credit approval, documentation, final pricing, equipment eligibility, transaction structure and closing conditions.

| 17B | 60-MONTH FINANCING CASHFLOW Appendix to Section 17 |
| --- | --- |

Months 1 through 25

| Month | Loan payment | Carbon credits | EVSE profit | Net monthly cashflow |
| --- | --- | --- | --- | --- |
| 1 | $5,566.56 | $3,640.99 | $2,239.19 | $313.63 |
| 2 | $5,566.56 | $3,640.99 | $2,239.19 | $313.63 |
| 3 | $5,566.56 | $3,640.99 | $2,239.19 | $313.63 |
| 4 | $5,566.56 | $3,640.99 | $2,239.19 | $313.63 |
| 5 | $5,566.56 | $3,640.99 | $2,239.19 | $313.63 |
| 6 | $5,566.56 | $3,640.99 | $2,239.19 | $313.63 |
| 7 | $5,566.56 | $3,640.99 | $2,239.19 | $313.63 |
| 8 | $5,566.56 | $3,640.99 | $2,239.19 | $313.63 |
| 9 | $5,566.56 | $3,640.99 | $2,239.19 | $313.63 |
| 10 | $5,566.56 | $3,640.99 | $2,239.19 | $313.63 |
| 11 | $5,566.56 | $3,640.99 | $2,239.19 | $313.63 |
| 12 | $5,566.56 | $3,640.99 | $2,239.19 | $313.63 |
| 13 | $5,566.56 | $3,640.99 | $2,519.09 | $593.53 |
| 14 | $5,566.56 | $3,640.99 | $2,519.09 | $593.53 |
| 15 | $5,566.56 | $3,640.99 | $2,519.09 | $593.53 |
| 16 | $5,566.56 | $3,640.99 | $2,519.09 | $593.53 |
| 17 | $5,566.56 | $3,640.99 | $2,519.09 | $593.53 |
| 18 | $5,566.56 | $3,640.99 | $2,519.09 | $593.53 |
| 19 | $5,566.56 | $3,640.99 | $2,519.09 | $593.53 |
| 20 | $5,566.56 | $3,640.99 | $2,519.09 | $593.53 |
| 21 | $5,566.56 | $3,640.99 | $2,519.09 | $593.53 |
| 22 | $5,566.56 | $3,640.99 | $2,519.09 | $593.53 |
| 23 | $5,566.56 | $3,640.99 | $2,519.09 | $593.53 |
| 24 | $5,566.56 | $3,640.99 | $2,519.09 | $593.53 |
| 25 | $5,566.56 | $3,640.99 | $2,833.98 | $908.41 |

| 17C | 60-MONTH FINANCING CASHFLOW Appendix to Section 17 |
| --- | --- |

Months 26 through 50

| Month | Loan payment | Carbon credits | EVSE profit | Net monthly cashflow |
| --- | --- | --- | --- | --- |
| 26 | $5,566.56 | $3,640.99 | $2,833.98 | $908.41 |
| 27 | $5,566.56 | $3,640.99 | $2,833.98 | $908.41 |
| 28 | $5,566.56 | $3,640.99 | $2,833.98 | $908.41 |
| 29 | $5,566.56 | $3,640.99 | $2,833.98 | $908.41 |
| 30 | $5,566.56 | $3,640.99 | $2,833.98 | $908.41 |
| 31 | $5,566.56 | $3,640.99 | $2,833.98 | $908.41 |
| 32 | $5,566.56 | $3,640.99 | $2,833.98 | $908.41 |
| 33 | $5,566.56 | $3,640.99 | $2,833.98 | $908.41 |
| 34 | $5,566.56 | $3,640.99 | $2,833.98 | $908.41 |
| 35 | $5,566.56 | $3,640.99 | $2,833.98 | $908.41 |
| 36 | $5,566.56 | $3,640.99 | $2,833.98 | $908.41 |
| 37 | $5,566.56 | $3,640.99 | $3,188.22 | $1,262.66 |
| 38 | $5,566.56 | $3,640.99 | $3,188.22 | $1,262.66 |
| 39 | $5,566.56 | $3,640.99 | $3,188.22 | $1,262.66 |
| 40 | $5,566.56 | $3,640.99 | $3,188.22 | $1,262.66 |
| 41 | $5,566.56 | $3,640.99 | $3,188.22 | $1,262.66 |
| 42 | $5,566.56 | $3,640.99 | $3,188.22 | $1,262.66 |
| 43 | $5,566.56 | $3,640.99 | $3,188.22 | $1,262.66 |
| 44 | $5,566.56 | $3,640.99 | $3,188.22 | $1,262.66 |
| 45 | $5,566.56 | $3,640.99 | $3,188.22 | $1,262.66 |
| 46 | $5,566.56 | $3,640.99 | $3,188.22 | $1,262.66 |
| 47 | $5,566.56 | $3,640.99 | $3,188.22 | $1,262.66 |
| 48 | $5,566.56 | $3,640.99 | $3,188.22 | $1,262.66 |
| 49 | $5,566.56 | $3,640.99 | $3,586.75 | $1,661.19 |
| 50 | $5,566.56 | $3,640.99 | $3,586.75 | $1,661.19 |

| 17D | 60-MONTH FINANCING CASHFLOW Appendix to Section 17 |
| --- | --- |

Months 51 through 60

| Month | Loan payment | Carbon credits | EVSE profit | Net monthly cashflow |
| --- | --- | --- | --- | --- |
| 51 | $5,566.56 | $3,640.99 | $3,586.75 | $1,661.19 |
| 52 | $5,566.56 | $3,640.99 | $3,586.75 | $1,661.19 |
| 53 | $5,566.56 | $3,640.99 | $3,586.75 | $1,661.19 |
| 54 | $5,566.56 | $3,640.99 | $3,586.75 | $1,661.19 |
| 55 | $5,566.56 | $3,640.99 | $3,586.75 | $1,661.19 |
| 56 | $5,566.56 | $3,640.99 | $3,586.75 | $1,661.19 |
| 57 | $5,566.56 | $3,640.99 | $3,586.75 | $1,661.19 |
| 58 | $5,566.56 | $3,640.99 | $3,586.75 | $1,661.19 |
| 59 | $5,566.56 | $3,640.99 | $3,586.75 | $1,661.19 |
| 60 | $5,566.56 | $3,640.99 | $3,586.75 | $1,661.19 |

| 18 | TERMS, PROPOSAL VALIDITY AND ACCEPTANCE |
| --- | --- |

Non-binding proposal disclaimer

This proposal is provided for evaluation, budgeting and scope-alignment purposes unless expressly incorporated into a definitive agreement. Pricing, schedule, incentives and projections are based on the information and assumptions identified in this document and may change following final engineering, site investigation, utility review, permitting, equipment availability, financing approval or other project-development findings. No contractual obligation arises until authorized representatives execute the applicable agreement, statement of work and commercial terms.

Revenue and incentive disclaimer

Utilization, revenue, energy cost, uptime, tax benefits, environmental credits and payback are projections rather than guarantees. Actual results will depend on driver demand, pricing, equipment availability, utility rates, operations, program rules and other factors outside Zero Impact Energy control. The client should obtain independent tax, legal and accounting advice.

Acceptance to proceed to definitive agreement

| Field | Entry |
| --- | --- |
| Client legal name | Jerome Jenkins |
| Title | Owner |
| Signature |  |
| Date |  |

This acknowledgment confirms the client’s intent to proceed with final scope development and contract preparation. It is not a notice to proceed unless expressly stated in the definitive agreement.